# Innovative Assignment Report

## Submitted by

Name: Parth Savaliya – 24BCE167 and Khush Pithva– 24BCE174

Branch: B.Tech CSE
Semester: 3rd
Subject: Digital Electronics (Innovative Assignment)

## Title

Design and Implementation of Binary Division Circuit Using Logisim

## Objective

The objective of this project is to design and implement a Binary Division Circuit using digital logic gates and flip-flops in Logisim software. The main goal is to simulate how binary division (similar to arithmetic division) can be performed using hardware components.

## Introduction

Binary division is one of the four basic arithmetic operations performed on binary numbers. It is similar to decimal division but uses only two digits — 0 and 1. In computer systems, arithmetic operations such as addition, subtraction, multiplication, and division are implemented using digital logic circuits. This project focuses on the hardware-level implementation of binary division using combinational and sequential circuits. The circuit divides a binary dividend by a divisor and produces the quotient and remainder outputs.

## Software Used

Logisim Evolution (Digital Logic Simulator)

## Components Used

1. AND Gates
2. OR Gates
3. XOR Gates
4. NOT Gates (Inverter)
5. MUX (Multiplexers)
6. Flip-Flops (for storing intermediate results)
7. Input Pins (for dividend and divisor bits)
8. Output Pins (for quotient and remainder)
9. Clock Signal (for synchronization in sequential operations)

## Circuit Description

The designed circuit performs binary division by repeatedly subtracting the divisor from the dividend and shifting bits accordingly — similar to the restoring division algorithm used in computer architecture.

- The dividend and divisor are given as binary inputs through input pins.
- The circuit uses a series of flip-flops and multiplexers to store and shift bits during each clock cycle.
- Subtraction is performed using XOR gates and Adders, while comparison and control are handled using AND/OR/NOT gates.
- The quotient bits are generated based on whether the divisor can be subtracted from the partial remainder.
- After the final iteration, the circuit displays both the quotient and remainder as output.

## Working Principle (Simplified)

1. Initialize dividend and divisor.
2. Align divisor with the most significant bit of dividend.
3. Subtract divisor from the current partial dividend.
4. If result is positive → write 1 in quotient and use the new remainder.
5. If result is negative → restore the value and write 0 in quotient.
6. Shift the divisor right and repeat the process for all bits.
7. At the end, the quotient and remainder are displayed as final outputs.

## Algorithm Used

Restoring Division Algorithm
- Step 1: Load dividend in register A and divisor in register B.
- Step 2: Shift left and subtract B from A.
- Step 3: If result ≥ 0 → set quotient bit = 1, else restore A and set quotient bit = 0.
- Step 4: Repeat for all bits.
- Step 5: After the final step, A holds the remainder, and quotient bits form the result.

## Simulation Results

The circuit was successfully simulated in Logisim Evolution.
- For test input Dividend = 1011 (11) and Divisor = 0101 (5),
  Quotient = 10 (2)
  Remainder = 1 (1)

The outputs were verified to be correct for multiple binary inputs, confirming that the logic and circuit design function properly.

## Applications

- Used in ALU (Arithmetic Logic Unit) design in processors.
- Used in digital calculators and embedded systems.
- Useful for educational and research projects on computer arithmetic.

## Conclusion

The Binary Division Circuit designed in Logisim successfully performs division between two binary numbers using logic gates, multiplexers, and flip-flops. The simulation results match the expected theoretical outcomes. This innovative assignment helped in understanding digital arithmetic circuits, algorithm-to-hardware translation, and sequential logic design.