# Binary Division Circuit: Detailed Summary with Examples

---

## 1. Components and Their Roles

| Component | Purpose in Circuit | Simple Meaning | Example |
| --- | --- | --- | --- |
| D Flip-Flop | Stores quotient and remainder bits after each step | Memory | If remainder is 0110 after a subtraction, the DFF stores it until the next clock tick |
| Multiplexer (MUX) | Chooses whether to subtract divisor or just shift remainder | Decision maker | If input 1 = remainder - divisor, input 0 = remainder unchanged; control signal chooses which passes forward |
| AND Gate | Controls signals or enables subtraction only when needed | Gatekeeper | Only allow subtraction when 'Enable' = 1, otherwise skip |
| XOR Gate | Flips bits for 2’s complement subtraction or compares bits | Bit flipper | Inverting divisor bits: 0011 becomes 1100 for subtraction; also checks if two bits differ |
| XNOR Gate | Checks if two bits are equal for control logic | Same-checker | Checks if remainder bits match 0000 after final subtraction |
| Adder | Performs subtraction using 2’s complement method | Calculator | To subtract 0011 from remainder 0110: take 2’s complement of 0011 → 1101, then add to 0110 → 0011 |
| Clock | Moves the circuit step by step | Timekeeper | Each clock toggle processes one dividend bit; 4-bit dividend → 5 toggles (extra for final result) |

---

## 2. Example: 4-bit Dividend ÷ 4-bit Divisor

* Dividend: 1101 (13 decimal)
* Divisor: 0011 (3 decimal)

Step-by-step process (5 clock toggles):

| Clock Toggle | Action | Values (Remainder / Quotient) |
| --- | --- | --- |
| 1 | Shift 1st bit of dividend | Remainder = 1 → cannot subtract, Quotient bit = 0 |
| 2 | Shift 2nd bit | Remainder = 11 → subtract divisor 11 → Remainder = 0, Quotient bit = 1 |
| 3 | Shift 3rd bit | Remainder = 0 → cannot subtract, Quotient bit = 0 |
| 4 | Shift 4th bit | Remainder = 01 → cannot subtract, Quotient bit = 0 |
| 5 | Final correction / store result | Remainder = 1, Quotient = 0100 |

---

## 3. Summary

**D Flip-Flop:** Stores quotient and remainder; example: storing remainder 0110

**MUX:** Decides subtraction or shift; example: choosing between R-D or just R

**AND:** Controls enable signals; example: only allow subtraction if 'Enable' = 1

**XOR:** Flips bits for subtraction; example: 0011 → 1100

**XNOR:** Checks equality of bits; example: remainder = 0000 after final step

**Adder:** Performs subtraction using 2’s complement; example: 0110 + 1101 = 0011

**Clock:** Moves process bit by bit; example: 4-bit dividend → 5 toggles

**Rule for clock toggles:** N-bit dividend → N + 1 toggles (extra for final correction and storing final result)