Report of

“Innovative Assignment”

For

2CS507CC23

Digital Electronics

B. Tech. Semester III

Report Prepared By

Khush Pithva 24BCE174

Parth Savalia 24BCE167

School of Technology

Institute of Technology

Nirma University

Ahmedabad 382481

# INDEX

| Sr No. | Description | Page No. |
| --- | --- | --- |
| 1 | About the Project | 3 |
| 2 | Inputs to BCD | 4 |
| 3 | BCD to Output | 11 |
| 4 | Logisim Implementation | 18 |
| 5 | Acknowledgment | 24 |
| 6 6 | Conclusion | 25 |

About the Project

We have designed an advanced Universal Code Converter, capable of transforming any 4-bit input into a range of commonly used coding formats, including Excess-3, 84-2-1, Gray, and Binary Coded Decimal (BCD).

Procedure

Step 1: Converting Inputs to BCD

- Gray to BCD

    - Truth Table

| Decimal | Input (Gray) |  |  |  | Output(BCD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I4 | I3 | I2 | I1 | B1 | B2 | B3 | B4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 4 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 5 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |
| 6 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 7 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 8 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |
| 9 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |

    - Karnaugh Map (Kmap)

| B1 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 0 | 0 |
| 0 1 | 0 | 0 | 0 | 0 |
| 1 1 | X | X | X | X |
| 1 0 | 1 | 1 | X | X |

| B2 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 0 | 0 |
| 0 1 | 1 | 1 | 1 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 0 | X | X |

| B3 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 1 | 1 |
| 0 1 | 0 | 0 | 1 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 0 | X | X |

| B4 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 1 | 1 | 0 |
| 0 1 | 0 | 1 | 1 | 0 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 1 | X | X |

- Excess-3 to BCD

- Truth Table

| Decimal | Input (EX-3) |  |  |  | Output(BCD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I4 | I3 | I2 | I1 | B1 | B2 | B3 | B4 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 3 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 4 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 5 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 6 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 7 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| 8 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| 9 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |

- K maps

| B1 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | X | X | 0 | X |
| 0 1 | 1 | 0 | 0 | 1 |
| 1 1 | 1 | X | X | X |
| 1 0 | 1 | 0 | 0 | 1 |

| B2 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | X | X | 0 | X |
| 0 1 | 0 | 1 | 0 | 1 |
| 1 1 | 0 | X | X | X |
| 1 0 | 0 | 1 | 0 | 1 |

| B3 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | X | X | 0 | X |
| 0 1 | 0 | 0 | 1 | 0 |
| 1 1 | 0 | X | X | X |
| 1 0 | 1 | 1 | 0 | 1 |

| B4 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | X | X | 0 | X |
| 0 1 | 0 | 0 | 0 | 0 |
| 1 1 | 1 | X | X | X |
| 1 0 | 0 | 0 | 1 | 0 |

- 84-2-1 to BCD

- Truth Table

| Decimal | Input (84-2-1) |  |  |  | Output(BCD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I4 | I3 | I2 | I1 | B1 | B2 | B3 | B4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 3 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 5 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 6 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
| 7 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 9 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |

- K maps

| B1 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | X | X | X |
| 0 1 | 0 | 1 | 1 | 0 |
| 1 1 | X | X | 1 | X |
| 1 0 | 0 | 1 | 1 | 0 |

| B2 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | X | X | X |
| 0 1 | 0 | 1 | 0 | 1 |
| 1 1 | X | X | 0 | X |
| 1 0 | 0 | 1 | 0 | 1 |

| B3 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | X | X | X |
| 0 1 | 1 | 0 | 0 | 0 |
| 1 1 | X | X | 0 | X |
| 1 0 | 0 | 1 | 1 | 1 |

| B4 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | X | X | X |
| 0 1 | 0 | 0 | 0 | 0 |
| 1 1 | X | X | 1 | X |
| 1 0 | 1 | 0 | 0 | 0 |

Final Equation

|  | BCD | GRAY | EX-3 | 84-2-1 |
| --- | --- | --- | --- | --- |
| B4 | I4 | I4 | I3I4+I4I2I1 | I3I4 + I4I2`I1` |
| B3 | I3 | I4⊕I3 | I1I2I3 + I1I2`I4 + I1`I3`I4 | I1`I2`I3 + I3`I1 + I3`I2 |
| B2 | I2 | I4⊕I3⊕I2 | I1 ⊕ I2 | I1 ⊕ I2 |
| B1 | I1 | I4⊕I3⊕I2⊕I1 | I1` | I1 |

Step 2: Converting BCD to Output

- BCD to Gray

- Truth Table

| Decimal | Input (BCD) |  |  |  | Output(Gray) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B4 | B3 | B2 | B1 | O1 | O2 | O3 | O4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 3 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 5 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 7 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 8 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |

- K maps

| O1 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 1 | 0 | 1 |
| 0 1 | 0 | 1 | 0 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 1 | X | X |

| O2 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 1 | 1 |
| 0 1 | 1 | 1 | 0 | 0 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 0 | X | X |

| O3 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 0 | 0 |
| 0 1 | 1 | 1 | 1 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 1 | 1 | X | X |

- BCD to EX-3

- Truth Table

| Decimal | Input (BCD) |  |  |  | Output(EX-3) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B4 | B3 | B2 | B1 | O1 | O2 | O3 | O4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 2 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 3 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 5 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 6 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 7 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 8 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |

- K maps

| O1 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 1 | 0 | 0 | 1 |
| 0 1 | 1 | 0 | 0 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 1 | 0 | X | X |

| O2 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 1 | 0 | 1 | 0 |
| 0 1 | 1 | 0 | 1 | 0 |
| 1 1 | X | X | X | X |
| 1 0 | 1 | 0 | X | X |

| O3 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 1 | 1 | 1 |
| 0 1 | 1 | 0 | 0 | 0 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 1 | X | X |

| O4 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 0 | 0 |
| 0 1 | 0 | 1 | 1 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 1 | 1 | X | X |

- BCD to 84-2-1

- Truth Table

| Decimal | Input (BCD) |  |  |  | Output(84-2-1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B4 | B3 | B2 | B1 | O1 | O2 | O3 | O4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
| 3 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 8 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |

- K maps

| O1 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 1 | 1 | 0 |
| 0 1 | 0 | 1 | 1 | 0 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 1 | X | X |

| O2 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 1 | 0 | 1 |
| 0 1 | 0 | 1 | 0 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 1 | X | X |

| O3 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 1 | 1 | 1 |
| 0 1 | 1 | 0 | 0 | 0 |
| 1 1 | X | X | X | X |
| 1 0 | 0 | 1 | X | X |

| O4 | 0 0 | 0 1 | 1 1 | 1 0 |
| --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 0 | 0 |
| 0 1 | 0 | 1 | 1 | 1 |
| 1 1 | X | X | X | X |
| 1 0 | 1 | 1 | X | X |

Final Equation

|  | BCD | GRAY | EX-3 | 84-2-1 |
| --- | --- | --- | --- | --- |
| O4 | B4 | B4 | B4+B3B1+B3B2 | B4+B3B1+B3B2 |
| O3 | B3 | B3+B4 | B1`B2`B3 + B3`B1 + B3`B2 | B1`B2`B3 + B3`B1 + B3`B2 |
| O2 | B2 | B2⊕B3 | B2⊙B1 | B1 ⊕ B2 |
| O1 | B1 | B1⊕B2 | B1` | B1 |
|  | S2` S1` 0 0 | S2` S1 0 1 | S2 S1` 1 0 | S2 S1 1 1 |

Step 3: Logisim Implementation

- Inputs to BCD

C2 and C1 are the selection lines which let user to decide the format of the input.

| C2’ C1’ | BCD |
| --- | --- |
| C2` C1 | Gray |
| C2 C1` | EX-3 |
| C2 C1 | 84-2-1 |

I4, I3, I2 and I1 are the input bits which is the input which is to be converted to desired format.

B4 = C2`.C1`.I4 + C2`.C1.I4 + C2.C1`.( I3.I4 + I4.I2.I1) + C2.C1(I3.I4 + I4.I2`.I1`)

B3 = C2`.C1`.I3 + C2`.C1(I4⊕I3) + C2.C1`(I1.I2.I3 + I1.I2`.I4 + I1`.I3`.I4) + C2.C1(I1`.I2`.I3 + I3`.I1 + I3`.I2)

B2 = C2`.C1`.I2 + C2`.C1(I4⊕I3⊕I2) + C2.C1`(I1⊕I2) + C2.C1(I1⊕I2)

B1 = C2`.C1`.I1 + C2`.C1(I4⊕I3⊕I2⊕I1) + C2.C1`.I1` + C2.C1.I1

- BCD to Output

S1 and S2 are the selection lines which let user decide the format of the output

| S2` S1` | BCD |
| --- | --- |
| S2` S1 | Gray |
| S2 S1` | EX-3 |
| S2 S1 | 84-2-1 |

B1, B2, B3 and B4 are the 4 BCD bits which is to be converted to the desired output.

O4 = S2`.B4 + S2.(B4+B3.B1+B3.B2)

O3= S2`.S1`.B3 + S2`.S1(B3+B4) + S2(B1`.B2`.B3 + B3`.B1 + B3`.B2)

O2= S2`.S1`.B2 + S2`.S1.(B2⊕B3) + S2.S1`(B2⊙B1) + S2.S1(B1⊕B2)

O1= S2`.S1`.B1 + S2`.S1`(B1⊕B2) + S2.S1.B1` + S2.S1.B1

- Final Circuit

Acknowledgment

We would like to express our sincere gratitude to everyonce who supported us and contributed to the successful completion of this project, “Universal Code Convertor”.

Firstly, we would like to thank our professor Mrs. Sharada Valiveti for her invaluable knowledge and encouragement for this project. Her teachings on fundamental of Digital Electronics and different code types helped us tremendously during this project.

This project has been an enlightening experience and we are thankful to all who supported directly or indirectly for the success of this project.

Conclusion

The "Universal Code Converter" project successfully demonstrates the design and implementation of a versatile digital system capable of converting any 4-bit input into multiple coded outputs, including Excess-3, 84-2-1, Gray, and Binary Coded Decimal (BCD). By addressing the need for efficient code conversion across digital systems, this converter simplifies the process of translating binary data into various formats, which is essential for compatibility in numerous applications.