NAME : Khush Pithva (24BCE174)

SUBJECT : DIGITAL ELECTRONICS (2CS507CC23)

PRACTICAL NO.: 6

AIM: Demonstrate the working of basic logic gates and verify it with Verilog HDL.

# AND GATE

CODE:

module and_gate(

input A,    input B,    output Y

);

assign Y = A & B;

endmodule module and_gate_tb;

reg tb_A;    reg tb_B;    wire tb_Y; and_gate dut(

.A(tb_A),

.B(tb_B),

.Y(tb_Y)

);

initial begin     tb_A = 1'b0; tb_B = 1'b0; #10; // A=0, B=0, wait 10 time units     tb_A = 1'b0; tb_B = 1'b1; #10; // A=0, B=1, wait 10 time units     tb_A = 1'b1; tb_B = 1'b0; #10; // A=1, B=0, wait 10 time units     tb_A = 1'b1; tb_B = 1'b1; #10; // A=1, B=1, wait 10 time units

$finish ;

end      initial      begin

$monitor("Time=%0t: A=%b, B=%b, Y=%b", $time, tb_A, tb_B, tb_Y);

end

endmodule OUTPUT:

# OR GATE

CODE:

module or_gate(

input A,    input B,    output Y

);

assign Y = A || B;

endmodule module or_gate_tb;

reg tb_A;    reg tb_B;    wire tb_Y; or_gate dut(    .A(tb_A),

.B(tb_B),

.Y(tb_Y)

);

initial begin     tb_A = 1'b0; tb_B = 1'b0; #10; // A=0, B=0, wait 10 time units     tb_A = 1'b0; tb_B = 1'b1; #10; // A=0, B=1, wait 10 time units     tb_A = 1'b1; tb_B = 1'b0; #10; // A=1, B=0, wait 10 time units     tb_A = 1'b1; tb_B = 1'b1; #10; // A=1, B=1, wait 10 time units

$finish ;     end      initial      begin

$monitor("Time=%0t: A=%b, B=%b, Y=%b", $time, tb_A, tb_B, tb_Y);

end

endmodule OUTPUT:

NAND GATE

CODE:

module nand_gate(

input A,    input B,    output Y

);

assign Y = ~(A & B);

endmodule module nand_gate_tb;

reg tb_A;    reg tb_B;    wire tb_Y; nand_gate dut(

.A(tb_A),

.B(tb_B),

.Y(tb_Y)

);

initial begin     tb_A = 1'b0; tb_B = 1'b0; #10; // A=0, B=0, wait 10 time units     tb_A = 1'b0; tb_B = 1'b1; #10; // A=0, B=1, wait 10 time units     tb_A = 1'b1; tb_B = 1'b0; #10; // A=1, B=0, wait 10 time units     tb_A = 1'b1; tb_B = 1'b1; #10; // A=1, B=1, wait 10 time units

$finish ;     end      initial      begin     $monitor("Time=%0t: A=%b, B=%b, Y=%b", $time, tb_A, tb_B, tb_Y);

end

endmodule OUTPUT:

# NOR GATE

CODE:

module nand_gate(

input A,    input B,    output Y

);

assign Y = ~(A & B);

endmodule module nand_gate_tb;

reg tb_A;    reg tb_B;    wire tb_Y; nand_gate dut(

.A(tb_A),

.B(tb_B),

.Y(tb_Y)

);

initial begin     tb_A = 1'b0; tb_B = 1'b0; #10; // A=0, B=0, wait 10 time units     tb_A = 1'b0; tb_B = 1'b1; #10; // A=0, B=1, wait 10 time units     tb_A = 1'b1; tb_B = 1'b0; #10; // A=1, B=0, wait 10 time units     tb_A = 1'b1; tb_B = 1'b1; #10; // A=1, B=1, wait 10 time units

$finish ;     end      initial      begin

$monitor("Time=%0t: A=%b, B=%b, Y=%b", $time, tb_A, tb_B, tb_Y);

end

endmodule OUTPUT:

# X-OR GATE

CODE:

module nand_gate(

input A,    input B,    output Y); assign Y = ~(A & B); endmodule module nand_gate_tb;

reg tb_A;    reg tb_B;    wire tb_Y; nand_gate dut(

.A(tb_A),

.B(tb_B),

.Y(tb_Y)

);

initial begin     tb_A = 1'b0; tb_B = 1'b0; #10; // A=0, B=0, wait 10 time units     tb_A = 1'b0; tb_B = 1'b1; #10; // A=0, B=1, wait 10 time units     tb_A = 1'b1; tb_B = 1'b0; #10; // A=1, B=0, wait 10 time units     tb_A = 1'b1; tb_B = 1'b1; #10; // A=1, B=1, wait 10 time units

$finish ;     end      initial      begin

$monitor("Time=%0t: A=%b, B=%b, Y=%b", $time, tb_A, tb_B, tb_Y);

end

endmodule OUTPUT:

X-NOR GATE

CODE:

module nand_gate(

input A,    input B,    output Y

);

assign Y = ~(A & B);

endmodule module nand_gate_tb;

reg tb_A;    reg tb_B;    wire tb_Y; nand_gate dut(

.A(tb_A),

.B(tb_B),

.Y(tb_Y)

);

initial begin     tb_A = 1'b0; tb_B = 1'b0; #10; // A=0, B=0, wait 10 time units     tb_A = 1'b0; tb_B = 1'b1; #10; // A=0, B=1, wait 10 time units     tb_A = 1'b1; tb_B = 1'b0; #10; // A=1, B=0, wait 10 time units     tb_A = 1'b1; tb_B = 1'b1; #10; // A=1, B=1, wait 10 time units

$finish ;     end      initial      begin     $monitor("Time=%0t: A=%b, B=%b, Y=%b", $time, tb_A, tb_B, tb_Y);

end

endmodule OUTPUT:

# NOT GATE

CODE:

module not_gate(

input A,    output Y

);

assign Y = ~(A);

endmodule module not_gate_tb;

reg tb_A;    wire tb_Y; not_gate dut(

.A(tb_A),

.Y(tb_Y)

);

initial begin     tb_A = 1'b0; #10; // A=0,wait 10 time units     tb_A = 1'b1; #10; // A=1,wait 10 time units

$finish ;     end      initial begin

$monitor("Time=%0t: A=%b, Y=%b", $time, tb_A, tb_Y);

end

endmodule OUTPUT :

NAME : KHUSH PITHVA (24BCE174)

SUBJECT : DIGITAL ELECTRONICS (2CS507CC23)

PRACTICAL NO.: 6

AIM: Design and Implementation of Half and Full Adder and Subtractor circuits.

# HALF ADDER

CODE:

module half_adder(

input A,     input B,     output Y,     output carry

);

assign Y = A ^ B; assign carry = A & B; endmodule module half_adder_tb; reg tb_A; reg tb_B; wire tb_Y; wire tb_carry; half_adder dut(

.A(tb_A),

.B(tb_B),

.Y(tb_Y),

.carry(tb_carry)

); initial begin tb_A = 1'b0; tb_B = 1'b0; #10;  tb_A = 1'b0; tb_B = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b0; #10;  tb_A = 1'b1; tb_B = 1'b1; #10;

$finish ; end

initial

begin

$monitor("Time=%0t: A=%b, B=%b, Y=%b, carry=%b", $time, tb_A, tb_B, tb_Y, tb_carry); end endmodule

OUTPUT :

# FULL ADDER

CODE:

module full_adder(     input A,     input B,     input C,     output Y,

output carry

);

assign Y = A ^ B ^ C; assign carry = (A & B) || (A ^ B) & C;

endmodule

module full_adder_tb; reg tb_A; reg tb_B; reg tb_C; wire tb_Y;

wire tb_carry;

full_adder dut(

.A(tb_A),

.B(tb_B),

.C(tb_C),

.Y(tb_Y),

.carry(tb_carry)

);

initial begin tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b0; #10;

tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b1; #10;

$finish ; end

initial begin

$monitor("Time=%0t: A=%b, B=%b, C=%b, Y=%b, carry=%b", $time, tb_A, tb_B, tb_C, tb_Y, tb_carry); end endmodule

OUTPUT :

# HALF ADDER

CODE :

module half_sub(

input A,     input B,     output Y,     output borrow

);

assign Y = A ^ B; assign borrow = ~A & B; endmodule module half_sub_tb; reg tb_A; reg tb_B; wire tb_Y; wire tb_borrow; half_sub dut( .A(tb_A),

.B(tb_B),

.Y(tb_Y),

.borrow(tb_borrow)

);

initial begin tb_A = 1'b0; tb_B = 1'b0; #10;  tb_A = 1'b0; tb_B = 1'b1; #10; tb_A = 1'b1; tb_B = 1'b0; #10;  tb_A = 1'b1; tb_B = 1'b1; #10;

$finish ;

end

initial

begin

$monitor("Time=%0t: A=%b, B=%b, Y=%b, borrow=%b", $time, tb_A, tb_B, tb_Y, tb_borrow);

End

endmodule

OUTPUT :

# FULL SUBSTRACTOR

CODE :

module full_sub(

input A,     input B,     input C,     output Y,     output borrow

);

assign Y = A ^ B ^ C; assign borrow = (~A & B) || (~A ^ B) & C; endmodule module full_sub_tb; reg tb_A; reg tb_B; reg tb_C; wire tb_Y; wire tb_borrow; full_sub dut( .A(tb_A),

.B(tb_B),

.C(tb_C),

.Y(tb_Y),

.borrow(tb_borrow)

); initial begin tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b1; #10;

$finish ; end

initial

begin

$monitor("Time=%0t: A=%b, B=%b, C=%b, Y=%b, borrow=%b", $time, tb_A, tb_B, tb_C, tb_Y, tb_borrow); end endmodule

OUTPUT :

NAME : KHUSH PITHVA (24BCE174)

SUBJECT : DIGITAL ELECTRONICS (2CS507CC23)

PRACTICAL NO.: 6

AIM: Design and Implementation of a combination circuit that converts the given BCD number into an equivalent 2421 number.

CODE:

module BCD(

input A,     input B,     input C,     input D,     output W,     output X,     output Y,     output Z

);

assign W = (B & D) || (B & C) || A; assign X = (B & ~D) || (B & C) || A; assign Y = (~B & C) || (B & ~C & D) || A ; assign Z = D ;

endmodule

module BCD_tb; reg tb_A; reg tb_B; reg tb_C;

reg tb_D; wire tb_W; wire tb_X; wire tb_Y; wire tb_Z;

BCD dut(

.A(tb_A),

.B(tb_B),

.C(tb_C),

.D(tb_D),

.W(tb_W),

.X(tb_X),

.Y(tb_Y),

.Z(tb_Z)

);

initial begin tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b0; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b1; #10; $finish ; end

initial

begin

$display("   BCD NUMBER       TO     2 4 2 1 NUMBER ");

$display("-------------------------------------");

$display("A   B   C   D    ||     W   X   Y   Z     ");

$display("-------------------------------------");

$monitor("%b   %b   %b   %b    ||     %b   %b   %b   %b    ", tb_A, tb_B, tb_C, tb_D, tb_W, tb_X, tb_Y, tb_Z); end endmodule

OUTPUT :

AIM: Design and Implementation of a combination circuit that converts a decimal digit from 84-2-1 code to 5421 code.

CODE:

module decimal(     input A,     input B,     input C,     input D,     output W,     output X,     output Y,

output Z

);

assign W = (A & ~C & ~D) || (A & B); assign X = (B & ~C & ~D) || (A & D) || (A & C); assign Y = (C ^ D) || (A & ~C);

assign Z = (A & ~C) || (~B & D) || (~A & D);

endmodule

module decimal_tb;

reg tb_A; reg tb_B; reg tb_C; reg tb_D; wire tb_W; wire tb_X; wire tb_Y;

wire tb_Z;

decimal dut( .A(tb_A),

.B(tb_B),

.C(tb_C),

.D(tb_D),

.W(tb_W),

.X(tb_X),

.Y(tb_Y),

.Z(tb_Z)

);

initial begin tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b0; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b1; #10;

$finish ; end

initial begin

$display("   8 4 -2 -1 NUMBER  TO     5 4 2 1 NUMBER ");

$display("-------------------------------------");

$display("A   B   C   D     ||     W   X   Y   Z     ");

$display("-------------------------------------");

$monitor("%b   %b   %b   %b     ||     %b   %b   %b   %b    ", tb_A, tb_B, tb_C, tb_D, tb_W, tb_X, tb_Y, tb_Z); end endmodule

OUTPUT :

AIM: Design and implementation of a combinational circuit that generates the square of a given 3 bit number.

CODE :

module square(

input X,     input Y,     input Z,     output A,     output B,     output C,     output D,     output E,     output F

);

assign A = X & Y; assign B = X & (~Y || Z); assign C = Z & (X ^ Y); assign D = Y & ~Z; assign E = 1'b0; assign F = Z;

endmodule module square_tb; reg tb_X; reg tb_Y; reg tb_Z; wire tb_A; wire tb_B; wire tb_C;

wire tb_D; wire tb_E; wire tb_F; square dut( .X(tb_X),

.Y(tb_Y),

.Z(tb_Z),

.A(tb_A),

.B(tb_B),

.C(tb_C),

.D(tb_D),

.E(tb_E),

.F(tb_F)

);

initial begin tb_X = 1'b0; tb_Y = 1'b0; tb_Z = 1'b0;  #10; tb_X = 1'b0; tb_Y = 1'b0; tb_Z = 1'b1;  #10;  tb_X = 1'b0; tb_Y = 1'b1; tb_Z = 1'b0;  #10; tb_X = 1'b0; tb_Y = 1'b1; tb_Z = 1'b1;  #10;  tb_X = 1'b1; tb_Y = 1'b0; tb_Z = 1'b0;  #10;  tb_X = 1'b1; tb_Y = 1'b0; tb_Z = 1'b1;  #10;  tb_X = 1'b1; tb_Y = 1'b1; tb_Z = 1'b0;  #10; tb_X = 1'b1; tb_Y = 1'b1; tb_Z = 1'b1;  #10;

$finish ;

end

initial

begin

$display("SQUARE OF A GIVEN 3-bit NUMBER "); $display("----------------------------------------------");

$display("X   Y   Z       ||      A   B   C   D   E   F    ");

$display("----------------------------------------------");

$monitor("%b   %b   %b       ||      %b   %b   %b   %b   %b   %b     ", tb_X, tb_Y, tb_Z, tb_A, tb_B, tb_C, tb_D, tb_E, tb_F); end endmodule

OUTPUT :

AIM: Design a 4 bit parity generator and checker.

CODE :

module parity(

input A,     input B,     input C,     input D,     output P,     output checker

);

assign P = ~(A ^ B ^ C ^ D); assign checker = ~(A ^ B ^ C ^ D ^ P);

endmodule

module parity_tb; reg tb_A; reg tb_B; reg tb_C; reg tb_D; wire tb_P; wire tb_checker;

parity dut(

.A(tb_A),

.B(tb_B),

.C(tb_C),

.D(tb_D),

.P(tb_P),

.checker(tb_checker)

);

initial begin tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b0; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b1; #10;  tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b0; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b1; #10;  tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b0; tb_D = 1'b1; #10; tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b0; tb_C = 1'b1; tb_D = 1'b1; #10; tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b0; tb_D = 1'b1; #10; tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b0; #10; tb_A = 1'b1; tb_B = 1'b1; tb_C = 1'b1; tb_D = 1'b1; #10;

$finish ;

end

initial

begin

$display("Parity Generator & Checker ");

$display("-----------------------------------------");

$display("A   B   C   D     ||    Parity   Checker    ");

$display("-----------------------------------------");

$monitor("%b   %b   %b   %b     ||      %b         %b     ", tb_A, tb_B, tb_C, tb_D, tb_P, tb_checker); end endmodule

OUTPUT :